Introduction to Embedded Systems

& IoT Device Design

Embedded Systems Task - 1

Mini Project: Smart Temperature Monitor

| Field | Details |
| --- | --- |
| Student Name | Soham |
| College | Walchand College of Engineering, Sangli |
| Department | Electronics Engineering |
| Task | Embedded Systems Task-1 |
| Project Type | Smart Temperature Monitor (Simulation) |
| Platform | Tinkercad Circuits (Arduino UNO) |
| Date | 14 June 2026 |

# 1. Introduction to Embedded Systems

## 1.1 What is an Embedded System?

An embedded system is a combination of computer hardware and software designed to perform a dedicated function or a specific set of functions within a larger system. Unlike general-purpose computers, embedded systems are purpose-built and optimized to execute predefined tasks with high reliability, real-time performance, and often under strict resource constraints such as limited memory, processing power, and energy.

Embedded systems typically include a microcontroller or microprocessor, input/output peripherals, sensors or actuators, and firmware programmed in low-level languages such as C or C++. They form the backbone of modern electronics, silently operating inside everyday devices from washing machines to spacecraft.

## 1.2 Real-World Applications

Embedded systems are ubiquitous across every major industry:

| Domain | Applications | Examples |
| --- | --- | --- |
| Smart Home | Automation & control | Smart thermostats, automated lights, security cameras |
| Automotive | Safety & engine control | ABS, airbags, ECU, parking sensors, GPS |
| Healthcare | Monitoring & diagnosis | ECG machines, pacemakers, insulin pumps, pulse oximeters |
| Robotics | Motion & sensing | Industrial arms, drones, autonomous vehicles |
| Consumer Electronics | User interaction | Smartphones, microwave ovens, washing machines |
| Aerospace | Navigation & control | Flight control systems, satellite transponders |
| Industrial IoT | Remote monitoring | SCADA systems, PLC-based factory automation |

## 1.3 Microcontroller vs Microprocessor

Understanding the key distinction between a microcontroller and a microprocessor is fundamental in embedded systems engineering:

| Feature | Microcontroller (MCU) | Microprocessor (MPU) |
| --- | --- | --- |
| Integration | CPU + RAM + ROM + I/O on single chip | CPU only; needs external RAM, ROM, peripherals |
| Cost | Low (₹50 – ₹500) | Higher (₹200 – ₹5000+) |
| Power Consumption | Very Low (mW range) | Higher (watts range) |
| Operating Speed | 8 MHz – 240 MHz | GHz range |
| Real-time Tasks | Excellent (dedicated hardware) | Requires RTOS for real-time |
| Applications | IoT sensors, appliances, robotics | Laptops, desktop PCs, servers |
| Examples | Arduino UNO (ATmega328P), ESP32, STM32 | Intel i5/i7, ARM Cortex-A series, Raspberry Pi 4 |
| Storage | Integrated Flash (2 KB – 4 MB) | External HDD/SSD required |

# 2. Component Study

## 2.1 Microcontrollers

Arduino UNO (ATmega328P)

The Arduino UNO is one of the most popular open-source microcontroller boards in the world, built around the ATmega328P 8-bit AVR microcontroller. It is the ideal starting point for beginners and professionals alike due to its extensive library support and community.

| Specification | Value |
| --- | --- |
| Microcontroller | ATmega328P |
| Operating Voltage | 5V |
| Digital I/O Pins | 14 (of which 6 provide PWM output) |
| Analog Input Pins | 6 |
| Flash Memory | 32 KB (0.5 KB used by bootloader) |
| SRAM | 2 KB |
| EEPROM | 1 KB |
| Clock Speed | 16 MHz |
| Communication | UART, SPI, I2C |

ESP32

The ESP32 is a powerful, low-cost dual-core microcontroller developed by Espressif Systems. It integrates both Wi-Fi (802.11 b/g/n) and Bluetooth (Classic + BLE) capabilities on a single chip, making it the go-to choice for IoT applications requiring wireless connectivity.

| Specification | Value |
| --- | --- |
| Processor | Xtensa dual-core LX6 (32-bit) |
| Clock Speed | Up to 240 MHz |
| Flash Memory | 4 MB (typical) |
| SRAM | 520 KB |
| Connectivity | Wi-Fi + Bluetooth 4.2 + BLE |
| GPIO Pins | 34 programmable pins |
| ADC Channels | 18 channels (12-bit) |
| Operating Voltage | 3.3V |
| Special Features | Deep sleep mode, touch sensors, Hall sensor |

Raspberry Pi Pico

The Raspberry Pi Pico is a low-cost, high-performance microcontroller board built around the RP2040 chip — Raspberry Pi's own silicon. It supports MicroPython and C/C++ and is designed for microcontroller-class applications rather than running a full operating system.

| Specification | Value |
| --- | --- |
| Processor | Dual-core Arm Cortex-M0+ @ 133 MHz |
| Flash Memory | 2 MB on-board |
| SRAM | 264 KB |
| GPIO Pins | 26 multi-function GPIO pins |
| Communication | 2× UART, 2× SPI, 2× I2C |
| ADC | 3× 12-bit ADC |
| USB | USB 1.1 host and device support |
| Programming | C/C++, MicroPython |
| Price | ~₹350 (Approx.) |

## 2.2 Basic Sensors

| Sensor | Working Principle | Common Use Cases |
| --- | --- | --- |
| DHT11/LM35 Temperature Sensor | Measures ambient temperature; DHT11 uses a capacitive humidity element + thermistor. LM35 outputs analog voltage proportional to temperature (10mV/°C) | Weather stations, HVAC systems, industrial monitoring |
| PIR Motion Sensor (HC-SR501) | Detects infrared radiation changes caused by moving warm objects using a pyroelectric sensor. Outputs a digital HIGH signal on motion detection | Security alarms, automatic lighting, occupancy detection |
| LDR (Light Dependent Resistor) | Resistance decreases as light intensity increases (photoconductivity). Used with a voltage divider circuit to output analog values | Automatic street lights, camera exposure, solar trackers |
| Ultrasonic Sensor (HC-SR04) | Emits ultrasonic pulses at 40 kHz and measures time for echo to return, calculating distance using speed of sound | Obstacle avoidance, level measurement, parking sensors |

# 3. Mini Project – Smart Temperature Monitor

## 3.1 Project Overview

The Smart Temperature Monitor is a basic IoT-sensing project that continuously reads temperature values from a sensor and displays them. When the temperature crosses a predefined threshold (30°C in this implementation), the system triggers an alert using a red LED and a buzzer, while a green LED stays ON when temperature is within the safe range.

## 3.2 Components Required

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Main microcontroller / brain of the system |
| LM35 Temperature Sensor | 1 | Reads analog temperature values (10mV/°C) |
| Red LED | 1 | Alert indicator — turns ON when temp > threshold |
| Green LED | 1 | Safe indicator — ON when temp is within safe range |
| Buzzer (Active) | 1 | Audible alert when temperature exceeds threshold |
| 220Ω Resistors | 2 | Current limiting for LEDs |
| Breadboard | 1 | Prototyping / circuit assembly |
| Jumper Wires | As needed | Electrical connections |

## 3.3 Working Principle

The LM35 temperature sensor outputs an analog voltage directly proportional to temperature in Celsius (10mV per degree Celsius). The Arduino's analog-to-digital converter (ADC) reads this voltage on pin A0, which provides a 10-bit resolution (0–1023 counts) over a 0–5V range.

The conversion formula is: Temperature (°C) = (ADC Value × 5.0 / 1023.0) × 100

The system operates on a simple threshold logic:

- If Temperature > 30°C: Red LED turns ON, Green LED turns OFF, Buzzer sounds continuously

- If Temperature ≤ 30°C: Green LED stays ON, Red LED turns OFF, Buzzer remains silent

- The Serial Monitor displays real-time temperature readings every 1 second

## 3.4 Circuit Connections

| Component Pin | Arduino UNO Pin |
| --- | --- |
| LM35 – VCC (+) | 5V |
| LM35 – GND (–) | GND |
| LM35 – Output (Vout) | A0 (Analog Pin 0) |
| Red LED – Anode (+) | Digital Pin 8 (via 220Ω resistor) |
| Red LED – Cathode (–) | GND |
| Green LED – Anode (+) | Digital Pin 9 (via 220Ω resistor) |
| Green LED – Cathode (–) | GND |
| Buzzer – Positive (+) | Digital Pin 10 |
| Buzzer – Negative (–) | GND |

## 3.5 Arduino Source Code (C++)

The following code implements the Smart Temperature Monitor. Each line is commented for clarity:

// ============================================================

//  Smart Temperature Monitor - Arduino UNO

//  Maincrafts Technology | Embedded Systems Task-1

//  Author  : Soham | Walchand College of Engineering

//  Platform: Arduino UNO + LM35 Sensor (Tinkercad Simulation)

// ============================================================

// --- Pin Definitions ---

const int TEMP_SENSOR_PIN = A0;  // LM35 output connected to A0

const int RED_LED_PIN     = 8;   // Red LED (alert) on digital pin 8

const int GREEN_LED_PIN   = 9;   // Green LED (safe) on digital pin 9

const int BUZZER_PIN      = 10;  // Buzzer on digital pin 10

// --- Threshold (degrees Celsius) ---

const float TEMP_THRESHOLD = 30.0;  // Alert if temp exceeds 30°C

// --- Setup: runs once on power-on ---

void setup() {

Serial.begin(9600);           // Initialize serial monitor at 9600 baud

pinMode(RED_LED_PIN,   OUTPUT);  // Set red LED pin as output

pinMode(GREEN_LED_PIN, OUTPUT);  // Set green LED pin as output

pinMode(BUZZER_PIN,    OUTPUT);  // Set buzzer pin as output

// Boot-up message

Serial.println("=== Smart Temperature Monitor ===");

Serial.println("Threshold: 30 degrees Celsius");

Serial.println("Monitoring started...");

Serial.println("--------------------------------");

}

// --- Loop: runs continuously ---

void loop() {

// Step 1: Read raw ADC value from LM35

int rawValue = analogRead(TEMP_SENSOR_PIN);

// Step 2: Convert ADC value to voltage (0–5V range, 10-bit ADC)

float voltage = rawValue * (5.0 / 1023.0);

// Step 3: Convert voltage to temperature

// LM35 produces 10mV per degree Celsius

float temperature = voltage * 100.0;

// Step 4: Print temperature to Serial Monitor

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" C");

// Step 5: Compare with threshold and trigger alerts

if (temperature > TEMP_THRESHOLD) {

// HIGH ALERT: Temperature exceeded threshold

digitalWrite(RED_LED_PIN,   HIGH);  // Red LED ON

digitalWrite(GREEN_LED_PIN, LOW);   // Green LED OFF

digitalWrite(BUZZER_PIN,    HIGH);  // Buzzer ON

Serial.println(">>> ALERT: High Temperature! <<<");

} else {

// SAFE: Temperature within normal range

digitalWrite(RED_LED_PIN,   LOW);   // Red LED OFF

digitalWrite(GREEN_LED_PIN, HIGH);  // Green LED ON

digitalWrite(BUZZER_PIN,    LOW);   // Buzzer OFF

Serial.println("Status: NORMAL - Temperature OK");

}

// Step 6: Wait 1 second before next reading

delay(1000);

}

## 3.6 Tinkercad Simulation Setup

To simulate this project on Tinkercad Circuits (https://www.tinkercad.com), follow these steps:

- Go to tinkercad.com → Sign up for a free account → Click 'Circuits' → 'Create new Circuit'

- From the component panel on the right, drag and drop: Arduino UNO R3, Breadboard (small), Temperature Sensor (TMP36 — closest to LM35 in Tinkercad), 2× LEDs (Red and Green), 2× 220Ω Resistors, 1× Piezo Buzzer

- Wire all components as per the Circuit Connections table in Section 3.4

- Click on 'Code' → Switch to 'Text' mode → Paste the Arduino code from Section 3.5

- Click 'Start Simulation' → Open Serial Monitor at bottom to see temperature values → Rotate the TMP36 sensor knob (in Tinkercad) to simulate temperature increase above 30°C and observe LED and Buzzer behavior

## 3.7 Expected Simulation Output

=== Smart Temperature Monitor ===

Threshold: 30 degrees Celsius

Monitoring started...

--------------------------------

Temperature: 24.40 C

Status: NORMAL - Temperature OK

Temperature: 27.85 C

Status: NORMAL - Temperature OK

Temperature: 31.20 C

>>> ALERT: High Temperature! <<<

Temperature: 35.60 C

>>> ALERT: High Temperature! <<<

Temperature: 28.90 C

Status: NORMAL - Temperature OK

# 4. Documentation & Report

## 4.1 System Working Summary

The Smart Temperature Monitor operates as a closed-loop monitoring system. The LM35/TMP36 analog temperature sensor continuously converts ambient temperature into a proportional analog voltage. This signal is fed into the Arduino's ADC, which digitizes it into a 10-bit value (0–1023). A mathematical conversion formula transforms this digital value back into a meaningful temperature in degrees Celsius.

The Arduino's firmware then applies threshold logic: comparing the measured temperature against a preset limit of 30°C. Based on this comparison, it activates appropriate output actuators — LEDs and a buzzer — to communicate the system status. The entire cycle repeats every 1 second, simulating continuous real-time monitoring.

## 4.2 Real-World Applications

The Smart Temperature Monitor concept, though implemented as a basic prototype, forms the foundation for numerous real-world applications:

| Application Area | Use Case | Scale |
| --- | --- | --- |
| Server Room Monitoring | Detect overheating of servers and trigger cooling fans or alarms to prevent data loss | Enterprise |
| Cold Chain Logistics | Monitor refrigerated trucks/containers for vaccines and perishables during transport | Industrial |
| Greenhouse Automation | Maintain optimal growing temperature for plants by controlling ventilation and heaters | Agricultural |
| Healthcare Equipment | Monitor incubator temperatures in NICUs, blood banks, and vaccine storage facilities | Medical |
| Industrial Ovens/Furnaces | Automated temperature control in manufacturing processes for precision quality | Manufacturing |
| Home Automation | Smart thermostat systems that adjust AC or heating based on room temperature | Residential |
| Forest Fire Detection | Sensor nodes deployed in forests to detect abnormal temperature spikes early | Environmental |

## 4.3 Advantages & Limitations

| Advantages | Limitations |
| --- | --- |
| Low cost — uses basic components under ₹500 total | LM35/TMP36 measures only ambient temperature; not suitable for very high-temp environments |
| Simple wiring and beginner-friendly implementation | Single threshold — no hysteresis; may cause rapid ON/OFF switching near threshold |
| Real-time monitoring with 1-second update intervals | No wireless connectivity — data cannot be sent to cloud without additional modules |
| Easily expandable with Wi-Fi module (ESP8266/ESP32) for IoT cloud integration | No data logging — readings are not stored permanently |
| Scalable: multiple sensors can be added for zone monitoring | Buzzer produces a constant tone — no variation in alarm urgency levels |

## 4.4 Future Scope & Enhancements

- Replace Arduino UNO with ESP32 to add Wi-Fi connectivity and send alerts to a mobile app via MQTT or HTTP

- Integrate an OLED/LCD display (e.g., 16×2 LCD with I2C) to show temperature locally without a computer

- Add data logging to an SD card or cloud platform (ThingSpeak, Blynk) for historical analysis

- Implement variable threshold — adjustable via a potentiometer or web interface

- Add DHT11/DHT22 sensor to simultaneously monitor humidity alongside temperature

- Implement PWM-controlled fan speed proportional to temperature rise

- Deploy as a wireless sensor node using LoRa or Zigbee for factory-wide monitoring

## 4.5 Conclusion

This Embedded Systems Task-1 project successfully demonstrates the fundamental concepts of embedded systems design through a practical IoT prototype — the Smart Temperature Monitor. The project covers all required deliverables: introduction to embedded systems theory, microcontroller and sensor component study, a working mini project with documented circuit connections, fully commented Arduino/C++ source code, and an analysis of real-world applications.

By implementing this project on the Tinkercad Circuits simulator, it has been validated without requiring physical hardware, making it accessible and reproducible. The project reinforces key concepts such as analog-to-digital conversion, threshold-based control logic, and output actuation — all essential skills for a Junior Embedded Engineer.

## 4.6 References

- Arduino Official Documentation: https://www.arduino.cc/reference/en/

- Tinkercad Circuits Simulator: https://www.tinkercad.com/circuits

- LM35 Datasheet — Texas Instruments: https://www.ti.com/product/LM35

- ATmega328P Datasheet — Microchip Technology

- NPTEL — Introduction to Embedded Systems: https://nptel.ac.in

- ESP32 Technical Reference Manual — Espressif Systems: https://docs.espressif.com